A&C New Control System

Space for MTA validation stamp

ITNS Tagging (if relevant)

| Originator | Document Title | Document Number |  |  |
| --- | --- | --- | --- | --- |
|  |  | NCS/00/I/COMMON----/ES/010 |  |  |
|  | Equipment Specification: Graphical Analyser | Originator Number |  |  |
|  | ECCN Codes | US Origin |  |  |
|  | EU EC: | No |  |  |
|  | US EC: |  |  |  |
|  | EU ANNEX VII: |  |  |  |
|  | EU ANNEX XXIII: |  |  |  |
| Confidentiality | Document Status | Rev. | Date | Sheet |
| C1 (Internal) | Released | D | 02/07/2026 | 1/10 |

Revision History

| Rev | Description of Change | Date |
| --- | --- | --- |
| A | First Issue | 13/10/2026 |

TABLE OF CONTENT

1.	Introduction	4

1.1.	Objective	4

1.2.	Field of Application	4

1.3.	References and related documents	4

1.4.	Terminology, definitions and abbreviations	4

1.5.	Requirements	4

2.	Input	5

2.1.	Source’ Features	5

2.2.	Source’ contents	5

2.3.	File’ Features	5

3.	Treatment	6

3.1.	Time Base definition	6

4.	Interface	7

4.1.	Main GUI	7

4.2.	File Tree view of contents	7

4.3.	Template	7

4.4.	Graphical tools	7

4.5.	Conditional processing	8

5.	Output	9

6.	Architecture	10

# Introduction

## Objective

Define the technical specification of Python tool devoted to NPI-NCS development based on technology Framatome TXS Compact. That consists of an application of graphical analyzer compatible with csv files produced by the Framatome system PTBG.

## Field of Application

Perturbographie Service unit. The present tool shall be located into the SU devoted to perturbographie treatment.

## References and related documents

| Item / Title | Reference / Version |
| --- | --- |
| Specification PTBG | D02-ARV-01-286-550 |
| ALSPA HMI V6 Visualiseur de Courbes Y=f(t) – Manuel de Référence | PTP09A42021 |
| TDS V1.1.0 User Manual |  |

## Terminology, definitions and abbreviations

| PTBG | Fonction PerTurBoGraphie |
| --- | --- |
| Data package | That define the current content of file. In case of multiple PLC, you could find all the csv defined as child of active data package. In case of single source, the data package shall highlight only one csv file. |
| Axle | Use from Python and representing a graphical area |
| Raster | It’s defining the time step |

## Requirements

| Requirement ID | Section of the document that verifies the requirement statement | Comment |
| --- | --- | --- |

# Input

## Source’ Features

| Upstream Req | Statement of the requirement | Ver |
| --- | --- | --- |
| REQ_COMMON_KH_2100 | The PLC can run at different frequencies. Preferred are 10 & 20ms, but the present tool can’t be disturbed by this aspect | A |
| REQ_COMMON_KH_2101 | The PLC can consist of a system composed by several PLC | A |
| REQ_COMMON_KH_2102 | The PLC can be synchronized or not | A |
| REQ_COMMON_KH_2103 | Each PLC will produce its own package of data with its own timestamp. | A |
| REQ_COMMON_KH_2104 | The collection of data from several sources can generate variation of frequencies, dephasing, or loss of data flow | A |
| REQ_COMMON_KH_2105 | Single or multiple files | A |

Source’ contents

| Upstream Req | Statement of the requirement | Ver |
| --- | --- | --- |
| REQ_COMMON_KH_2200 | Binary signal | A |
| REQ_COMMON_KH_2201 | Analog signal type : float | A |
| REQ_COMMON_KH_2201 | Binary group signal | A |

## File’ Features

| Upstream Req | Statement of the requirement | Ver |
| --- | --- | --- |
| REQ_COMMON_KH_2300 | The preferred format shall be ‘csv’ | A |
| REQ_COMMON_KH_2301 | Separator shall be a comma ‘,’ | A |
| REQ_COMMON_KH_2302 | Text format UTF8 | A |
| REQ_COMMON_KH_2303 | No additional meta data | A |
| REQ_COMMON_KH_2304 | First column devoted to time base | A |
| REQ_COMMON_KH_2305 | Time stamp configuration: | A |
| REQ_COMMON_KH_2306 | Hh:mn:s… micro, nanos… | A |
| REQ_COMMON_KH_2307 | Time counter | A |
| REQ_COMMON_KH_2308 | First row devoted to column’s header | A |
| REQ_COMMON_KH_2309 | Composed by PLC’s variables names | A |
| REQ_COMMON_KH_2310 | Maximal size xxxMo | A |

# Architecture

By commodity with the LINUX SUSE environment the solution PYTHON is the preferred way to develop the present tool. However, to permit the continuous checking and improvement of the tool, we shall require several features as constraint to respect into the global architecture of the tool.

| Upstream Req | Statement of the requirement | Ver |
| --- | --- | --- |
| REQ_COMMON_KH_3000 | Final tool shall be compatible with SLES - SUSE 15 SP7: ‘SUSE Linux Enterprise Server 15SP7’ at least | A |
| REQ_COMMON_KH_3001 | Final tool shall be compatible with Python 3.6 at least | A |
| REQ_COMMON_KH_3002 | MAIN.py script shall be devoted only to generate the GUI interface. | A |
| REQ_COMMON_KH_3003 | All action realized by the tool shall be best as possible externalized into a dedicated sub script or class. | A |
| REQ_COMMON_KH_3004 | All additional packages or library implemented shall be documented and strictly aligned with the kernel version. | A |
| REQ_COMMON_KH_3005 | Compatibility mechanism between different OS Shall be documented. | A |
| REQ_COMMON_KH_3006 | Code source shall be commented | A |
| REQ_COMMON_KH_3007 | No useless function hold into the scripts | A |

# Treatment

## Time Base definition

| Upstream Req | Statement of the requirement | Ver |
| --- | --- | --- |
| REQ_COMMON_KH_4100 | The present tool shall define the time raster from the source and control check: • Period value • Period stability • Period continuous increase | A |
| REQ_COMMON_KH_4101 | The present tool shall be capable to find and highlight the inconsistent time base like missing data or wrong phasing | A |
| REQ_COMMON_KH_4102 | A dedicated graphical axle shall be devoted to detail the time base conversion and status. It shall be automatically present on each graphical template and call: Time Base Monitor. | A |
| REQ_COMMON_KH_4103 | The time base monitor can be hidden by the issuer only if no warning or error. | A |
| REQ_COMMON_KH_4104 | The Time Base Monitor shall be automatically present into all reports. If removed for arrangement and indication shall appear to confirm the good construction or Time Base. | A |
| REQ_COMMON_KH_4105 | In case of disturbance in the time base analysis, the treatment can’t be locked or unstable. | A |
| REQ_COMMON_KH_4106 | In case of disturbance in the time base analysis, the user can specify the normal raster with an associated diagnosis | A |
| REQ_COMMON_KH_4107 | Constitute Absolute timestamp. Clock Jitter ignored | A |

# Interface

## Main GUI

| Upstream Req | Statement of the requirement | Ver |
| --- | --- | --- |
| REQ_COMMON_KH_5100 | Top bar with main menu path | A |
| REQ_COMMON_KH_5101 | Specific ‘Information’ section including: - Tool version - History of changes - Sources - Help file | A |
| REQ_COMMON_KH_5102 | Bottom bar summarizing: - User - Date - Activity log - Tool version | A |
| REQ_COMMON_KH_5103 | Left area will contain the data tree viewer | A |
| REQ_COMMON_KH_5104 | Right area will be the main and wider section devoted to graphical analysis. | A |
| REQ_COMMON_KH_5105 | Tree area can be hidden | A |
| REQ_COMMON_KH_5106 | Graphical area can manage several tab to hold different arrangements | A |

## File Tree view of contents

| Upstream Req | Statement of the requirement | Ver |
| --- | --- | --- |
| REQ_COMMON_KH_5200 | Top Tree level correspond to the data package | A |
| REQ_COMMON_KH_5201 | First Child level will be devoted to the csv file… from one source or several. | A |
| REQ_COMMON_KH_5202 | Into each csv file, the tree shall detail all the variables contained with associated type Bool or Float. | A |
| REQ_COMMON_KH_5203 | Into each csv file, the tree shall detail the dedicated time base | A |

## Template

| Upstream Req | Statement of the requirement | Ver |
| --- | --- | --- |
| REQ_COMMON_KH_5300 | Several templates shall be proposed including different arrangement of Axle | A |

## Graphical tools

| Upstream Req | Statement of the requirement | Ver |
| --- | --- | --- |
| REQ_COMMON_KH_5400 | Vertical single Cursor: can be activated by axle | A |
| REQ_COMMON_KH_5401 | Vertical single Cursor: hold time stamp value and cross value of selected curve | A |
| REQ_COMMON_KH_5402 | Vertical single Cursor: tracking reading mode adding the list of crossed value from all the axle’s curves into a right popup | A |
| REQ_COMMON_KH_5403 | Double Vertical Cursor: Time stamp is mandatory and automated for each cursor | A |
| REQ_COMMON_KH_5404 | Double Vertical Cursor: Time stamp is mandatory and automated for each cursor | A |
| REQ_COMMON_KH_5405 | Global cursor = global synchronization of axles. Cursor appear over all axles and moving in same time | A |
| REQ_COMMON_KH_5406 | Horizontal cursor: only applicable to analog signals | A |
| REQ_COMMON_KH_5407 | Horizontal cursor: can be conditioned by additional bool signal to realize conditional processing. | A |
| REQ_COMMON_KH_5408 | Cursor X/Y: used for correlation control check - Two cursors are activated or the cross point only can be highlighted | A |

## Conditional processing

| Upstream Req | Statement of the requirement | Ver |
| --- | --- | --- |
| REQ_COMMON_KH_5500 |  | A |

# Output

| Upstream Req | Statement of the requirement | Ver |
| --- | --- | --- |
| REQ_COMMON_KH_6000 | The present tool shall be capable to generate a pdf report. | A |
| REQ_COMMON_KH_6001 | Report consist into a template containing Header and graphical area | A |
| REQ_COMMON_KH_6002 | Report, can be generated automatically or step by step. | A |
| REQ_COMMON_KH_6003 | Automatic Report consist into a single edition of active graphical area to pdf file | A |
| REQ_COMMON_KH_6004 | Manual step by step report generation consist into a sequential generation on order depending of graphical active tab. Permitting to push global view, and customize analysis, cursor, X/Y cloud… | A |
| REQ_COMMON_KH_6005 | Header shall contain following details: - Tool Version - Issuer - Date - Source File(s) - Project Name & Unit - Project file reference or Hash | A |